# Workshop Manual: Herb Trough & Bird Feeder

Comprehensive Construction Standards, Material Cut Lists, and Assembly Protocols.

# Project 1: X-Leg Herb Trough (v2)

# Core Design Rules & Critical Lock-ins

- One Saw Setting for All Four Feet: Cut every 33° foot end off the exact same saw stop setting. Cutting feet individually will cause the stand to rock and wobble.

- Lock the X Crossing: A single center screw allows the legs to scissor. Always secure each joint with 3–4 screws or pass a solid 6 mm or 10 mm bolt through the crossing.

- Standard 300 mm Top Gauge: Set leg tops exactly 300 mm apart from inside corner to inside corner to ensure both X frames are identical.

- Leg Lap Joint: The upper leg lies flat across the lower leg without notched joints, creating distinct inside and outside leg orientations.

- Wall Offset Standards: Back wall planks sit 23 mm proud behind the outside leg, while front planks mount flush/proud to match the leg profile.

# Herb Trough Cut List

| Part | Qty | Length (mm) | Width (mm) | Thick (mm) | Construction Notes |
| --- | --- | --- | --- | --- | --- |
| Leg | 4 | 700 | 80 | 23 | One end cut at 33°; all four cut at an identical saw setting. |
| Cross Plank (Wall) | 8 | 900 | 80 | 23 | 4 planks front, 4 planks back. |
| Side Piece | 6 | Scribed | 80 | 23 | 3 per end; scribed to fit the V-frame. |
| Bottom Base Pair | 2 | 900 | 80 | 23 | Front edge cut to sit in V; back trimmed 23 mm. |

# Material & Hardware Requirements

- Timber: Untreated pine or hardwood. Untreated wood is mandatory because this is a food bed.

- Fasteners: 35 mm part-threaded screws for crossing joints and side courses.

- Bolts (Optional): 6 mm or 10 mm stainless or coated bolts through each crossing for permanent rigidity.

- Finish: Woodoc wood oil applied to the exterior surfaces and foot bases only.

- Spacer Blocks: Small filler blocks fitted at the shoulder gap between angled leg heads and plank undersides.

# Step-by-Step Herb Trough Assembly

## Stage 1: Constructing the X-Leg Frames

Cut one end of each 700 mm leg at 33° using a fixed stop block. Mark the center point of each leg at 350 mm along length and 40 mm across width. Clamp a straight guide plank to your workbench to reference the 33° foot cuts flush against it. Cross the upper leg straight over the lower leg, set the top spread to 300 mm inside-corner to inside-corner, drill a pilot hole, and drive a 35 mm part-threaded screw through the center. Lock the joint using 3–4 extra screws or a central bolt. Build two mirrored frame assemblies.

## Stage 2: Standing the Frames & Setting Base Planks

Stand the two X frames parallel on a level surface. Cut the bottom front 900 mm plank edge at a 22.5° angle so it nests snugly into the V-bottom. Because the back plank overlaps the front plank, trim 23 mm off the back plank's width so both top edges sit perfectly level.

## Stage 3: Stacking Wall Planks

Stack 4 planks high on the front and 4 planks high on the back, resting each plank on the course below. Align all planks flush with the left and right X frames. The back wall stack rests 23 mm proud behind the outside leg to clear the leg offset. Verify that both front and back walls reach identical height, then screw the planks securely into the X frames.

## Stage 4: Scribed End Pieces

To seal the trough ends, start at the bottom of the box inside the V-angle. Set the first side piece at a 33 degree angle so it sits snugly into the V-bottom starting from the base of the box. Scribe and fit another 2 side pieces above it, working upwards toward the top rim (totaling 3 pieces per end). Each piece must be screwed and glued in place using weatherproof wood glue and screws to create a strong, durable side structure. Repeat this procedure for the opposite end frame.

# Project 2: The Offcut Bird Feeder (Mark Piece 01-v1)

# Overview & Structural Specifications

A hanging platform bird feeder constructed from untreated Baltic pine. It utilizes a single tool-free joint with no glue in the main structural frame, allowing quick disassembly for cleaning.

# Bird Feeder Cut List

| Part | Qty | Length (mm) | Width (mm) | Thick (mm) | Construction Notes |
| --- | --- | --- | --- | --- | --- |
| End Wall / Gable | 2 | 198 | 145 | 18 | Top edge cut to 25° pitch (146 mm eave, 180 mm ridge). |
| Tray Floor | 1 | 340 | 145 | 18 | Base tray platform. |
| Tray Long Wall | 2 | 304 | 50 | 18 | Shallow wall keeps seed contained above wet line. |
| Roof Panel | 2 | 380 | 100 | 18 | Mitred at ridge peak. |
| Ridge Cap | 1 | 380 | 20 | 20 | Top protective ridge cover. |
| Ridge Beam (Clamp) | 1 | 304 | 20 | 20 | Clamping bar mounted beneath roof peak. |

# Materials, Hardware & Finish

- Timber: Untreated Baltic pine (38 × 150 × 900 mm resawn to 18 mm boards). Pressure-treated or CCA-treated wood must never touch bird contact surfaces.

- Threaded Rod Joint: M6 stainless steel threaded rod (350 mm long) with 2 wing nuts and 2 washers.

- Fasteners: Approximately 22 stainless steel screws (4 × 40 mm) for tray walls.

- Finish: Apply 2 coats of Woodoc sealer on all exterior faces. Leave the tray interior completely bare.

# Water, Service & Installation

- Weather Protection: Roof overhangs measure 15 mm at eaves and 20 mm at gables. The 50 mm walls hold bird seed dry above the wet line.

- Drainage: Drill 4 × 8 mm drainage holes in the tray floor.

- Tool-Free Cleaning: Undo 2 wing nuts to remove the M6 rod; the roof and tray lift apart instantly without tools.

- Hanging: Drill an 8 mm hole through the center of the ridge cap. Suspend with cord or chain approximately 2 m above ground level and 3 m clear of surrounding trees for squirrel jumping clearance.

# Structural Diagram & Schematic Visuals

- Elevation Dimensions: Roof pitch is set to 25°. Eave height is 146 mm; ridge height is 180 mm. Tray overall length is 340 mm, total roof length is 380 mm.

- Clamping Mechanism: An M6 stainless threaded rod passes horizontally through both end gables and the central ridge clamping beam. Tightening wing nuts on outer gable faces clamps the entire assembly solidly without glue.

- Note: User pictures can be supplied if needed for the feeder schematics.